Part 1

Two signals were created in MATLAB. An array was created to store the digits of each students reference number (2,0,2,0,0,9,3,3,4 for Andrew Law and 2,0,2,0,3,2,1,1,7 for Frank Conway. Then ‘repmat()’ was used to repeat the sequence 900 times, with the signals stored in x1 (for Andrew – referenced as ‘1’) and x2 (for Frank – referenced as ‘2’ throughout). The mean of each signal was taken and subtracted from them, which removed the DC component. The sampling frequency was calculated by averaging the concatenated last 4 digits of each student reference number.

The time domain plot of each signal (only 3 cycles) is shown below in Figures xx & xx:

The signals were transformed into the frequency domain using the ‘fft()’. The length was set to 1024 as this was greater than the number of samples, hence a more accurate spectrum would be produced due to the number of frequency bins equal to nfs/N (although not really required as trying to make a longer transition band anyways). It was also set to 1024 as this is a power-of-two value, which can force the FFT computation to be more efficient.

The frequency domain plots of signals x1  and x2 are shown below in Figures xx and xx:

Note the spectrum plots make sense as there is a spectral replication from 0→fs/2 & fs/2→fs, due to being periodic at integer multiples of fs/2.

The filters were designed by analysing the plots in Figures xx and xx. Since both spectrums had their strongest component occurring at 1906.8 Hz (962.2 for x1 and 652.7 for x2), the strongest component of x1 and the second strongest component of x2 were chosen to be removed. Both signals required a band pass filter since both were not the lowest frequency components, with x1’s filter (‘filter_a’) removing 1906.8 Hz and x2’s filter (‘filter_b’) removing 1274.9 Hz.

They were both designed using filterDesigner.

filter_a characteristics:

- Fpass1 = 1500 Hz

- Fstop1 = 1850 Hz

- Fstop2 = 1950 Hz

- Fpass2 = 2300 Hz

filter_a characteristics:

- Fpass1 = 750 Hz

- Fstop1 = 1150 Hz

- Fstop2 = 1250 Hz

- Fpass2 = 1650 Hz

Note: For filter design, target was to keep the transition band as good as possible – ratio of transition band to fs – can’t alter fs but can have longer transition → less sharp ratio → less coeffs.

The time domain, magnitude and phase response of filter_a is shown below in Figures xx to xx, with Figures xx to xx for filter_b:

The following is filter_a, then filter_b, pole-zero diagrams before and after quantisation.

For both filters, before and after quantisation has occurred:

- Poles are inside the unit circle ∴ both systems (filters) are stable – parks algorithm ∴ dissnae matter

- Poles are at origin ∴ filters are indeed FIR filters

- Zeros are outside the unit circle ∴ filters are not minimum phase systems

- # poles = # zeros ∴ systems are causal

The signals x1 and x2 were filtered with filter_a and filter_b, respectively, using ‘conv()’. The time domain plot is shown below in Figures xx and xx:

As shown below in Figures xx and xx, filter_a successfully removed the 1.9 kHz component and filter_b successfully removed the 1.2 kHz, both without impacting any other components.

Show the difference!

conv1,2,3,4,5,6,7,8=5,16,34,60,61,52,32

Analytical conclusion – first update then final update to show it is same as MATLAB .